TRONTEK

LiEV LiFePO4 Battery — 51V 105Ah (5.355 kWh) Phase-II

Model TK LiEV-51105 (Phase-II)  |  51 V / 105 Ah  |  5.36 kWh  |  Lithium Ferro Phosphate

Online Product Listing Sheet — E-Rickshaw (3W) EV Battery

# 1. Product details

| Attribute | Value |
| --- | --- |
| Brand | TRONTEK® (LiEV / Lithium Ferro series) |
| Product name | Trontek LiEV LiFePO4 EV Battery 51V 105Ah (5.355 kWh) Phase-II |
| Model name | TK LiEV-51105 (Phase-II) (51V / 105Ah) |
| Category | 3-Wheeler / E-Rickshaw Lithium Battery / EV Battery |
| Product type | Electric vehicle traction battery pack (lithium iron phosphate) |
| Generic name | Lithium Iron Phosphate (LiFePO4) EV battery pack |
| Battery cell composition | Lithium Iron Phosphate (LiFePO4 / LFP) |
| Voltage | 51 Volts (nominal) |
| Battery capacity | 105 Ah (5.36 kWh) |
| Number of batteries | 1 × battery pack |
| Net quantity | 1 Count |
| Recommended uses | Electric 3-wheelers / e-rickshaws and e-loaders running on a 48V system; OEM fitment or lead-acid replacement with compatible controller and Trontek charger |
| Compatibility | 48VV EV drivetrain with 58.4V charging profile |
| Connector | Output cable connector (charge + motor power leads) |
| Enclosure | Powder-coated metal enclosure with orange top cover, dual carry handles and output cable glands |
| Colour | Black with orange top |
| Country of origin | India |
| Warranty | 3 years or 1200 cycles, whichever is earlier |

## Key features

- Attractive cycle life — 2000 cycles at 0.5C, 25°C

- Extended safety performance and double safety protection

- Wide operating temperature range; unrivalled high-temperature performance

- Green energy without metal contaminant

- High capacity, steady output voltage, low self-discharge

- Withstands very high levels of vibration and shock

- Intelligent BMS with independent cell balancing

## Safety characteristics (as tested)

- Over-charge / over-discharge: withstands both — no fire, no explosion, continues to work

- Short circuit: withstands short circuit — no fire, no explosion

- Nail puncture: withstands nail penetration — no fire, no explosion

- Thermal shock: withstands thermal shock — no fire, no explosion

## Description

The Trontek TK LiEV-51105 (Phase-II) is a 51V 105Ah (5.36 kWh) lithium iron phosphate traction battery built for electric 3-wheelers (e-rickshaws and e-loaders). LFP chemistry gives the safest lithium option on the road — no thermal runaway, no fire on puncture or short — with 2000+ cycle life and steady output voltage across the full discharge. The pack is optimised by its battery management system (BMS), which monitors every cell to protect against over-charge, over-discharge, over-current and short circuit, and balances cells independently for safe and accurate operation. Trontek is an established Indian battery manufacturer with 1 GWh production capacity and in-house R&D and testing.

# 2. Technical specifications

## Electrical characteristics

| Model | TK LiEV-51105 (Phase-II) |
| --- | --- |
| Nominal voltage | 51 V |
| Nominal capacity | 105 Ah |
| Battery pack energy | 5.36 kWh (5355 Wh) |
| Impedance (max. at 1000 Hz) | ≤ 20 mΩ |
| Expected cycle life | 2000 cycles at 0.5C, 25°C, within operating range |

## Operation conditions

| Charge method | Constant Current / Constant Voltage (CC-CV) |
| --- | --- |
| Max. charge voltage | 58.4 V |
| Standard charge current | 16 A |
| Max. charge current | 50 A |
| Continuous discharge current | 25 A |
| Peak instant discharge current | 75 A |
| Peak instant discharge time | 5 s |
| Discharge cut-off voltage | 43.2 V |
| Discharge temperature | −20°C to 60°C |
| Storage temperature | 15°C to 35°C |

## Mechanical characteristics

| Height | 280 ±2 mm |
| --- | --- |
| Width | 380 ±2 mm |
| Length | 585 ±2 mm |
| Weight | ~55 kg |

## BMS / PCM protection functions

Trontek LFP battery is optimised through its BMS — monitoring cells to protect against over-charge, over-discharge and short circuit, with independent balancing on every pack.

| Item | Content | Criterion |
| --- | --- | --- |
| Over charge protection | Over charge detection voltage | 3.7 ±0.025 V |
|  | Over charge release voltage | 3.6 ±0.025 V |
|  | Maximum charge voltage | 3.7 ±0.05 V |
|  | Standard charge current | ≤ 16 A |
| Over discharge protection | Over discharge detection voltage | 2.7 ±0.025 V |
|  | Over discharge detection delay time | 0.5 – 1.5 s |
|  | Over discharge release voltage | 2.6 ±0.025 V |
| Over discharge current protection | Over current detection current | 121 ±30 A |
|  | Detection delay time | 200 ms |
|  | Release condition | Cut load |
|  | Maximum continuous current | ≤ 96 A |
| Short circuit protection | Detection condition | Exterior short circuit (333 A) |
|  | Detection delay time | 200 µs |
|  | Release condition | Cut short circuit |

# 3. Frequently asked questions

Q1. Which electric 3-wheelers (e-rickshaws and e-loaders) is this battery compatible with?

Any 48VV-system e-rickshaw whose controller accepts a 51V nominal LFP pack with a 58.4V charge voltage and 43.2V cut-off. Confirm connector type (Output cable connector (charge + motor power leads)) and charger rating with iTarang before purchase; controller and charger must match.

Q2. What range will I get?

The pack stores 5.36 kWh. A typical e-rickshaw consumes 45–65 Wh/km, so expect roughly 82–119 km per full charge depending on passenger load, speed, terrain and motor rating.

Q3. How long does it take to charge?

At the standard charge current (16 A) a full charge from cut-off takes about 6.5–7 (16 A) / 2–2.5 (50 A fast) hours. Use only a Trontek-approved LFP charger with a 58.4V CC-CV profile; a lead-acid charger will damage the pack.

Q4. Is LFP safer than the NMC batteries in the news for fires?

Yes. LFP has a far higher thermal-runaway threshold and does not release oxygen when abused. This pack has been tested for over-charge, short circuit, nail puncture and thermal shock with no fire and no explosion, and the BMS cuts load on over-current, over-discharge and short circuit.

Q5. How many years will the battery last?

Rated for 2000 cycles at 0.5C and 25°C. At one full charge per day that is roughly 5–6 years of daily use before capacity drops to end-of-life, versus about 1–1.5 years for lead-acid EV batteries.

Q6. Can I use it in hot Indian summers and in the rain?

Discharge is rated −20°C to 60°C; the BMS throttles or cuts charging outside safe limits. Powder-coated metal enclosure with orange top cover, dual carry handles and output cable glands — mount inside the vehicle's battery compartment and keep the connector dry. Store between 15°C and 35°C.

Q7. Does it need maintenance?

No. No water topping, no acid, no terminal corrosion. Keep the connector clean and avoid leaving the pack at 0% for long periods.

Q8. What is the warranty?

Warranty: 3 years or 1200 cycles, whichever is earlier. Warranty is void if a non-approved charger is used or the pack is opened.

Q9. What happens at end of life?

The pack is fully recyclable. Return it through iTarang's battery buyback / take-back programme under the Battery Waste Management Rules, 2022. Never dispose of it in household waste.

# 4. Manufacturer details

| Manufacturer | Trontek (Trontek Electronics Pvt. Ltd. — confirm exact legal name and registered address) |
| --- | --- |
| Location | India — 1 GWh production capacity, in-house R&D and testing |
| Website | www.trontek.com |
| Email | info@trontek.com |
| Country of origin | India |

# 5. Marketer / seller details

| Marketed & sold by | iTarang Network Private Limited |
| --- | --- |
| Address | Unitech Business Zone, Sector 50, Gurugram, Haryana |
| Customer care | 9971027907 |
| Email | [insert customer-care email] |
| GSTIN | 06AALFI7813E1ZE |
| Grievance officer | Mr. Chirag Garg — 9971027907 |

# 6. Product size

| Dimensions (L × W × H) | 585 × 380 × 280 mm (±2 mm) |
| --- | --- |
| In centimetres | 58.5 × 38.0 × 28.0 cm |

# 7. Product weight

| Net weight | ~55 kg |
| --- | --- |
| Shipping weight | ~59.00 kg (net + 4 kg packaging) |

# 8. Product price

| Energy (V × Ah) | 51 V × 105 Ah = 5355 Wh (5.36 kWh) |
| --- | --- |
| MRP (inclusive of all taxes) @ ₹25 / Wh | ₹ 1,33,875 |
| Selling price @ ₹18 / Wh | ₹ 96,390 |
| Discount | 28% off MRP (₹ 37,485) |
| HSN code | 8507 60 00 (lithium-ion accumulators) |
| GST | 18% (verify current rate) |

MRP must be printed on the pack and match the listing.