TRONTEK

LiEV LiFePO4 Battery — 51V 45Ah (2.3 kWh)

Model TK-LiFe-5145  |  51 V / 45 Ah  |  2.29 kWh  |  Lithium Ferro Phosphate

Online Product Listing Sheet — 2-Wheeler EV Battery

# 1. Product details

| Attribute | Value |
| --- | --- |
| Brand | TRONTEK® (LiEV / Lithium Ferro series) |
| Product name | Trontek LiEV LiFePO4 EV Battery 51V 45Ah (2.3 kWh) |
| Model name | TK-LiFe-5145 (51V / 45Ah) |
| Category | 2-Wheeler Lithium Battery / EV Battery |
| Product type | Electric vehicle traction battery pack (lithium iron phosphate) |
| Generic name | Lithium Iron Phosphate (LiFePO4) EV battery pack |
| Battery cell composition | Lithium Iron Phosphate (LiFePO4 / LFP) |
| Voltage | 51 Volts (nominal) |
| Battery capacity | 45 Ah (2.29 kWh) |
| Number of batteries | 1 × battery pack |
| Net quantity | 1 Count |
| Recommended uses | Electric 2-wheelers (scooters / motorcycles) and light EVs running on a 48VV system; OEM replacement or retrofit with compatible controller and charger |
| Compatibility | 48VV EV drivetrain with 57.6V charging profile |
| Connector | Top-mounted connector with buzzer and breather |
| Enclosure | Extruded aluminium enclosure with handle, buzzer and breather valve |
| Colour | Grey/black enclosure |
| Country of origin | India |

## Key features

- Attractive cycle life — 2000 cycles at 0.5C, 25°C

- Extended safety performance and double safety protection

- Wide operating temperature range; unrivalled high-temperature performance

- Green energy without metal contaminant

- High capacity, steady output voltage, low self-discharge

- Withstands very high levels of vibration and shock

- Intelligent BMS with independent cell balancing

## Safety characteristics (as tested)

- Over-charge / over-discharge: withstands both — no fire, no explosion, continues to work

- Short circuit: withstands short circuit — no fire, no explosion

- Nail puncture: withstands nail penetration — no fire, no explosion

- Thermal shock: withstands thermal shock — no fire, no explosion

## Description

The Trontek TK-LiFe-5145 is a 51V 45Ah (2.29 kWh) lithium iron phosphate traction battery built for electric 2-wheelers. LFP chemistry gives the safest lithium option on the road — no thermal runaway, no fire on puncture or short — with 2000+ cycle life and steady output voltage across the full discharge. The pack is optimised by its battery management system (BMS), which monitors every cell to protect against over-charge, over-discharge, over-current and short circuit, and balances cells independently for safe and accurate operation. Trontek is an established Indian battery manufacturer with 1 GWh production capacity and in-house R&D and testing.

# 2. Technical specifications

## Electrical characteristics

| Model | TK-LiFe-5145 |
| --- | --- |
| Nominal voltage | 51 V |
| Nominal capacity | 45 Ah |
| Battery pack energy | 2.29 kWh (2295 Wh) |
| Impedance (max. at 1000 Hz) | ≤ 15 mΩ |
| Expected cycle life | 2000 cycles at 0.5C, 25°C, within operating range |

## Operation conditions

| Charge method | Constant Current / Constant Voltage (CC-CV) |
| --- | --- |
| Max. charge voltage | 57.6 V |
| Standard charge current | 0.3C (≈13.5 A) |
| Max. charge current | 0.5C (≈22.5 A) |
| Continuous discharge current | 45 A (max. 1.5C) |
| Peak instant discharge current | 90 A |
| Peak instant discharge time | 5 s |
| Discharge cut-off voltage | 44.8 V |
| Discharge temperature | −20°C to 60°C |
| Storage temperature | 15°C to 35°C |

## Mechanical characteristics

| Height | 356 ±2 mm |
| --- | --- |
| Width | 165 ±2 mm |
| Length | 230 ±2 mm |
| Weight | ~20 kg |

## BMS / PCM protection functions

Trontek LFP battery is optimised through its BMS — monitoring cells to protect against over-charge, over-discharge and short circuit, with independent balancing on every pack.

| Item | Content | Criterion |
| --- | --- | --- |
| Over charge protection | Over charge detection voltage | 3.6 ±0.025 V |
|  | Over charge release voltage | 3.5 ±0.025 V |
|  | Maximum charge voltage | 3.6 ±0.05 V |
|  | Standard charge current | ≤ 45 A |
| Over discharge protection | Over discharge detection voltage | 2.8 ±0.025 V |
|  | Over discharge detection delay time | ≤ 115 – 173 ms |
|  | Over discharge release voltage | 2.9 ±0.01 V |
| Over discharge current protection | Over current detection current | 121 ±30 A |
|  | Detection delay time | 32 ±16 ms |
|  | Release condition | Cut load |
|  | Maximum continuous current | ≤ 90 A |
| Short circuit protection | Detection condition | Exterior short circuit (850 A) |
|  | Detection delay time | 200 µs |
|  | Release condition | Cut short circuit |

# 3. Frequently asked questions

Q1. Which electric 2-wheelers is this battery compatible with?

Any 48VV-system electric scooter or motorcycle whose controller accepts a 51V nominal LFP pack with a 57.6V charge voltage and 44.8V cut-off. Confirm connector type (Top-mounted connector with buzzer and breather) and charger rating with iTarang before purchase; controller and charger must match.

Q2. What range will I get?

The pack stores 2.29 kWh. A typical e-scooter consumes 25–35 Wh/km, so expect roughly 66–92 km per full charge depending on rider weight, speed, terrain and motor rating.

Q3. How long does it take to charge?

At the standard charge current (0.3C (≈13.5 A)) a full charge from cut-off takes about 3.5–4 hours. Use only a Trontek-approved LFP charger with a 57.6V CC-CV profile; a lead-acid charger will damage the pack.

Q4. Is LFP safer than the NMC batteries in the news for fires?

Yes. LFP has a far higher thermal-runaway threshold and does not release oxygen when abused. This pack has been tested for over-charge, short circuit, nail puncture and thermal shock with no fire and no explosion, and the BMS cuts load on over-current, over-discharge and short circuit.

Q5. How many years will the battery last?

Rated for 2000 cycles at 0.5C and 25°C. At one full charge per day that is roughly 5–6 years of daily use before capacity drops to end-of-life, versus about 1–1.5 years for lead-acid EV batteries.

Q6. Can I use it in hot Indian summers and in the rain?

Discharge is rated −20°C to 60°C; the BMS throttles or cuts charging outside safe limits. Extruded aluminium enclosure with handle, buzzer and breather valve — mount inside the vehicle's battery compartment and keep the connector dry. Store between 15°C and 35°C.

Q7. Does it need maintenance?

No. No water topping, no acid, no terminal corrosion. Keep the connector clean and avoid leaving the pack at 0% for long periods.

Q8. What is the warranty?

Warranty period: [insert Trontek warranty terms — typically expressed in months or cycles]. Warranty is void if a non-approved charger is used or the pack is opened.

Q9. What happens at end of life?

The pack is fully recyclable. Return it through iTarang's battery buyback / take-back programme under the Battery Waste Management Rules, 2022. Never dispose of it in household waste.

# 4. Manufacturer details

| Manufacturer | Trontek (Trontek Electronics Pvt. Ltd. — confirm exact legal name and registered address) |
| --- | --- |
| Location | India — 1 GWh production capacity, in-house R&D and testing |
| Website | www.trontek.com |
| Email | info@trontek.com |
| Country of origin | India |

# 5. Marketer / seller details

| Marketed & sold by | iTarang Network Private Limited |
| --- | --- |
| Address | Unitech Business Zone, Sector 50, Gurugram, Haryana |
| Customer care | 9971027907 |
| Email | [insert customer-care email] |
| GSTIN | 06AALFI7813E1ZE |
| Grievance officer | Mr. Chirag Garg — 9971027907 |

# 6. Product size

| Dimensions (L × W × H) | 230 × 165 × 356 mm (±2 mm) |
| --- | --- |
| In centimetres | 23.0 × 16.5 × 35.6 cm |

# 7. Product weight

| Net weight | ~20 kg |
| --- | --- |
| Shipping weight | ~24.00 kg (net + 4 kg packaging) |

# 8. Product price

| Energy (V × Ah) | 51 V × 45 Ah = 2295 Wh (2.29 kWh) |
| --- | --- |
| MRP (inclusive of all taxes) @ ₹25 / Wh | ₹ 57,375 |
| Selling price @ ₹18 / Wh | ₹ 41,310 |
| Discount | 28% off MRP (₹ 16,065) |
| HSN code | 8507 60 00 (lithium-ion accumulators) |
| GST | 18% (verify current rate) |

MRP must be printed on the pack and match the listing.