TRONTEK

Powercube 2.7 — Home Battery Storage

Model TK25100  |  25.6 V / 105 Ah  |  2.688 kWh  |  LiFePO4

Online Product Listing Sheet

# 1. Product details

| Attribute | Value |
| --- | --- |
| Brand | TRONTEK® |
| Product name | Powercube 2.7 Home Battery Storage |
| Model name | TK25100 (25.6V/105Ah) |
| Generic name | Lithium Iron Phosphate inverter / home energy storage battery |
| Battery cell composition | Lithium Iron Phosphate (LiFePO4 / LFP) |
| Voltage | 25.6 Volts |
| Battery capacity | 105 Ah (2.688 kWh) |
| Number of batteries | 1 × 24V battery required |
| Net quantity | 1 Count |
| Recommended uses | Residential energy storage with hybrid / solar / lithium-compatible inverters; home backup power; storing excess PV generation; peak-demand supply; off-grid use |
| Inverter compatibility | Lithium-ready inverters with 24V battery input and 29.2V charging profile; compatible with major inverter brands |
| Installation | Wall or floor mounted (indoor, IP20) |
| Colour | White with black display strip |
| Country of origin | India |

## Key features

- High safety standard LFP (LiFePO4) cell chemistry

- Intelligent BMS — over-charge, over-discharge, over-current, short-circuit and over-temperature protection

- Long-life design supported by efficient cell equalization technology

- Compact wall/floor-mount design, easy installation

- Works with an existing photovoltaic system or a new hybrid inverter

- >4000 cycles at 80% DOD; 12-year design life at 35°C

## Description

The Trontek Powercube 2.7 is a residential energy storage system that acts as a power source, stores excess electricity generation, helps go off-grid, lowers power cost, minimises power loss, fulfils peak power demand and improves the home power network. Connect it to an existing photovoltaic system or install a new one with a hybrid inverter to become energy independent. The built-in intelligent BMS protects and monitors the lithium pack for prolonged cyclic life. Trontek is an established Indian battery manufacturer with 1 GWh production capacity, in-house R&D and testing.

# 2. Technical specifications

| Parameter | Value |
| --- | --- |
| Model | TK25100 |
| Type | 25.6V / 105Ah |
| Rated voltage | 25.6 V |
| Rated capacity (0.5CA) | 105 Ah |
| Energy | 2.688 kWh |
| Cell configuration | 8S1P |
| Cell voltage | 3.2 V |
| Voltage range | 21.6 – 29.2 V ±0.2 V |
| Charging voltage | 29.2 V ±0.2 V |
| Charging mode | CC/CV |
| Ah efficiency | >95% |
| Max. charge current | 0.5C (≈52.5 A) |
| Max. discharge current | 1C (≈105 A) |
| Design life @35°C (Class I & II area) | 12 years |
| Cycle life (80% DOD, 25°C, 0.5C charge / 0.5C discharge) | >4000 cycles |
| Charging temperature | 0°C to +45°C |
| Discharging temperature | −10°C to +60°C |
| Storage temperature & time | 1 year @20°C / 6 months @30°C / 3 months @40°C |
| Protection class | IP20 |
| Dimensions (L × W × H) | 362 × 132 × 354 mm ±2 mm |
| Weight | 20.4 kg |

# 3. Frequently asked questions

Q1. Will this work with my existing inverter?

Yes, if your inverter supports lithium (LFP) batteries with a 24V battery input and a 29.2V charging voltage. Most modern hybrid and "lithium-ready" inverters do. Older lead-acid-only inverters need a lithium charging profile or a compatible replacement.

Q2. How is this different from a lead-acid / tubular inverter battery?

LFP delivers 4000+ cycles versus roughly 800–1200 for tubular, 80–100% usable depth of discharge versus about 50%, around one-third the weight, no water topping, no acid fumes and faster charging. Upfront cost is higher; cost per cycle is lower.

Q3. How long will it run my home?

The battery stores 2.688 kWh. Divide by your load. Example: 300 W of fans, lights and Wi-Fi runs for about 8 hours at ~90% usable capacity.

Q4. Does it need maintenance?

No. There is no water topping, terminal cleaning or venting. The built-in BMS handles balancing and protection.

Q5. Is it safe to use indoors?

Yes. LFP is the most thermally stable lithium chemistry and the BMS protects against over-charge, over-discharge, over-current, short-circuit and over-temperature. The unit is IP20, so install indoors in a ventilated spot away from water.

Q6. Can I add a second battery later?

Check with the seller before paralleling; packs must be the same model and firmware. Never mix with lead-acid or other lithium brands.

Q7. What is the warranty and expected life?

Design life of 12 years at 35°C and more than 4000 cycles at 80% DOD. Warranty period: [insert Trontek warranty terms].

Q8. What happens to the battery at end of life?

It is recyclable. Return it through the seller's take-back programme under the Battery Waste Management Rules, 2022. Never dispose of it in household waste.

# 4. Manufacturer details

| Manufacturer | Trontek (Trontek Electronics Pvt. Ltd. — confirm exact legal name and registered address) |
| --- | --- |
| Location | India — 1 GWh production capacity, in-house R&D and testing |
| Website | www.trontek.com |
| Email | info@trontek.com |
| Country of origin | India |

# 5. Marketer / seller details

| Marketed & sold by | iTarang Network Private Limited |
| --- | --- |
| Address | Unitech Business Zone, Sector 50, Gurugram, Haryana |
| Customer care | 9971027907 |
| Email | [insert customer-care email] |
| GSTIN | 06AALFI7813E1ZE |
| Grievance officer | Mr. Chirag Garg — 9971027907 |

# 6. Product size

| Dimensions (L × W × H) | 362 × 132 × 354 mm (±2 mm) |
| --- | --- |
| In centimetres | 36.2 × 13.2 × 35.4 cm |

# 7. Product weight

| Net weight | 20.4 kg |
| --- | --- |
| Shipping weight | 24.4 kg (net + 4 kg packaging) |

# 8. Product price

| MRP (inclusive of all taxes) | ₹ [insert] |
| --- | --- |
| Selling price | ₹ [insert] |
| HSN code | 8507 60 00 (lithium-ion accumulators) |
| GST | 18% (verify current rate) |

MRP must be printed on the pack and match the listing.